Training Aid to Assist in Developing Smoke Containment Systems

– Elevator Smoke Control Systems Testing

- Normal HVAC Mode - Baseline

  - Verify the construction team responsibilities for system installation, startup, and Testing, Adjusting, and Balancing (TAB) of the HVAC system is complete.

  - Verify that all related building construction is complete.

    - Stairwells shall be complete with all doors and exit hardware in their final condition.

  - Verify the HVAC system is in normal operation without any calls for an event.

  - Identify and verify all smoke barrier doors serving the stairway and associated vestibules are closed.

    - Verify that all door hardware has been fitted.

    - Test each door in sequence to verify it closes and latches correctly.

    - Verify Door Sweeps are flush with ground level.

  - Measure and record the pressure differential across all stairway and vestibule doors.

  - Measure and record the door force required to open all stairway and vestibule doors

- Smoke Control Mode for Elevator Smoke Control Systems- Event

  - Verify the construction team responsibilities for system installation, startup, and Testing, Adjusting, and Balancing (TAB) of the HVAC system and Smoke Control System is complete.

  - Verify that all related building construction is complete.

  - If the elevator pressurization system is used in combination with a zoned smoke control system, the elevator pressurization system shall be considered a zone.

  - Each elevator lobby in an enclosed elevator lobby pressurization system shall be considered as one zone.

  - Activate the Elevator Smoke Control Systems for the zone being tested.

    - Testing shall not generate a false call to the fire department

  - Verify the operation of all applicable fans, dampers, doors, and related equipment.

    - Fans: Confirm 1.5 times the number of required belts with a minimum of 2 (IFC 909.10.5)

    - Fans: Confirm Actual Current Draw and a minimum Service Factor of 1.15. (IFC 909.10.5)

  - HVAC systems shall be off unless designed to be in operation during an event.

    - Verify operational status of HVAC unit matches the Sequence Of Operations (SOP).

  - Verify that all elevator cars in the tested shaft return to the recall floor, and remain there with the doors open for the duration of the test.

  - Verify all elevator doors are closed, with the exception of the door on the recall floor, if designed to be open per the Sequence of Operations (SOP).

  - Measure and record the pressure differential across each elevator door.

    - Confirm pressure differential is at or above the minimum pressure differential.

    - Clearly indicate high and low pressure side of smoke barrier or door

  - If the elevator is designed to move during elevator pressurization, the previous steps (except step h) shall be repeated during these conditions.

  - Adjustment

    - Adjust the fan speed or static setpoint, if required, to change the shaft pressurization to meet specified requirements.

  - Repeat and record all readings following any adjustment.

  - Report the actual test conditions and results to the AHJ and engineer of record.

    - Deficiencies will require a clear plan of action and temporary accommodations.

  - Ensure that the SCS systems are deactivated and the associated HVAC systems are returned to their normal operation.

This document is intended to be used solely as an aide when developing the methods, procedures, and forms used in the inspection of Smoke Control Systems (SCS).  It is the responsibility of each contractor, supervisor, and technician to ensure that the methods, procedures, and forms used meet the requirements of the local mechanical and fire codes. Furthermore, it is the responsibility of the SCS Supervisor or contractor to submit the methods, procedures, and forms that it drafts directly to the Authority Having Jurisdiction (AHJ) for approval, prior to performing the actual work.  The National Energy Management Institute Committee makes no representations, whatsoever, that drafting procedures or forms based on this document will meet that requirement of local mechanical and fire codes and expressly disclaims any liability or responsibility regarding the use of this document.