Training Aid to Assist in Developing Smoke Containment Systems

– Force Testing

- Identify and verify position of all doors and openings, that will remain open during an event.

- Identify and verify position of all doors and openings, that will remain closed during an event.

  - Verify that all door hardware has been fitted.

  - Test each door in sequence to verify it closes and latches correctly.

  - Verify Door Sweeps are flush with ground level.

- Activate the Smoke Containment System

  - Testing shall not generate a false call to the fire department

- Verify operational status of HVAC unit matched sequence of operations.

- Verify the operation of all applicable fans, dampers, doors, and related equipment.

  - Fans: Confirm 1.5 times the number of required belts with a minimum of 2 (IFC 909.10.5)

  - Fans: Confirm Actual Current Draw and a minimum Service Factor of 1.15. (IFC 909.10.5)

- Measure and record the door force needed to open each egress door.

  - Confirm required force to open the door is at or below the maximum allowed door force.

- Repeat test until all alarm inputs have been activated.

- Report the actual test conditions and results to the AHJ and engineer of record.

  - Deficiencies will require a clear plan of action and temporary accommodations.

- Ensure that the SCS systems are deactivated and the associated HVAC systems are returned to their normal operation.

This document is intended to be used solely as an aide when developing the methods, procedures, and forms used in the inspection of Smoke Control Systems (SCS).  It is the responsibility of each contractor, supervisor, and technician to ensure that the methods, procedures, and forms used meet the requirements of the local mechanical and fire codes. Furthermore, it is the responsibility of the SCS Supervisor or contractor to submit the methods, procedures, and forms that it drafts directly to the Authority Having Jurisdiction (AHJ) for approval, prior to performing the actual work.  The National Energy Management Institute Committee makes no representations, whatsoever, that drafting procedures or forms based on this document will meet that requirement of local mechanical and fire codes and expressly disclaims any liability or responsibility regarding the use of this document.